Duet Module Interface Specification

for a

Marantz DV7600 Disc Player

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Channels	5

Command Control	7

Command Feedback	9

Device Notes	11

Programming Notes	11

Adding Functions to Modules	12

Commands to the device	12

Responses from the device	12

LIST OF TABLES

Table 1 - Virtual Device Channel Events	6

Table 2 – Send Command Definitions	8

Table 3 - Command Feedback Definitions	10

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 01-31-06 | SP | Initial release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Marantz DV7600. The Marantz DV7600 supports an RS-232 serial protocol. The interface was tested using version 1.01 of the firmware. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#10-889-04. The wiring diagram for this cable is as follows:

AMX NXI	AMX NI 		Marantz DV7600 (Female DB9)

(Gnd) 1	(Gnd) 5  __________	(Gnd) 5

(Rx)   2	(Rx)   2  __________	(Tx)   2

(Tx)   3	(Tx)   3  __________	(Rx)   3

## Overview

The COMM module translates between the standard interface described below and the disc device serial protocol. It parses the buffer for responses from the disc device, sends strings to control the disc device, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

Marantz DV7600

Disc Player

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX Marantz DV7600 module, the programmer must perform the following steps:

- Define the virtual device ID that the Marantz DV7600 COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file Marantz_DV7600.TP4 and module ( Marantz_DV7600_UI.axs) have been created for testing.

- The Duet Marantz DV7600 module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the disc device to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

DEFINE_DEVICE

dvDEVICE   =  5001:1:0	// Marantz DV7600, Serial Port

dvTP             =  10001:1:0 	// Touch Panel

vdvDEVICE =  41001:1:0	// Virtual Device

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

VOLATILE INTEGER nTP_BUTTONS[] = {

1,				// POWER ON

2, 				// POWER OFF

3				// VIDEO MUTE

}

VOLATILE INTEGER nTXT_BTN[] = {

1

}

DEFINE_START

DEFINE_MODULE 'Marantz_DV7600_Comm_v1_0_0' COMM1(vdvDEVICE, dvDEVICE)

DEFINE_MODULE 'Marantz_DV7600_UI'	TP1(vdvDEVICE, dvTP, nTP_BUTTONS, nTXT_BTN)

Upon initialization the AMX Comm module will communicate with the disc device and information will be exchanged.

## Channels

The UI module controls the disc device via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

Note: An ‘*’ indicates an extension to the standard API.

| Channel | Description |
| --- | --- |
| 1 | PULSE: Play |
| 2 | PULSE: Stop |
| 3 | PULSE: Pause Note: Pressing Pause after the first time will cause the DVD to “Step” (appears to be the same functionality as “Frame”) |
| 4 | PULSE: Next |
| 5 | PULSE: Previous |
| 6 | PULSE: Scan Forward |
| 7 | PULSE: Scan Reverse |
| 9 | PULSE: Cycle Power |
| 10 | PULSE: 0 Digit Button |
| 11 | PULSE: 1 Digit Button |
| 12 | PULSE: 2 Digit Button |
| 13 | PULSE: 3 Digit Button |
| 14 | PULSE: 4 Digit Button |
| 15 | PULSE: 5 Digit Button |
| 16 | PULSE: 6 Digit Button |
| 17 | PULSE: 7 Digit Button |
| 18 | PULSE: 8 Digit Button |
| 19 | PULSE: 9 Digit Button |
| 20 | PULSE: +10 Button |
| 21 | PULSE: Enter Button |
| 27 | ON: Set Power On |
| 28 | ON: Set Power Off |
| 44 | PULSE: Menu Button |
| 45 | PULSE: Move Menu Cursor Up |
| 46 | PULSE: Move Menu Cursor Down |
| 47 | PULSE: Move Menu Cursor Left |
| 48 | PULSE: Move Menu Cursor Right |
| 49 | PULSE: Select Menu Item |
| 50 | PULSE: Exit Button |
| 66 | PULSE: SETUP Button |
| 80 | PULSE: Clear Button |
| 84 | PULSE: Dimmer Button |
| 99 | PULSE: Display Button |
| 100 | PULSE: Subtitle Button |
| 104 | PULSE: Return Button |
| 106 | PULSE: Page Up Button |
| 107 | PULSE: Page Down Button |
| 111 | PULSE: Program Button |
| 112 | PULSE: AB Repeat Button |
| 115 | PULSE: Top Menu Button |
| 116 | PULSE: Zoom Button |
| 117 | PULSE: Angle Button |
| 118 | PULSE: Audio Button |
| 120 | PULSE: Cycle the disc tray open and closed |
| 124 | PULSE: Cycle the random state |
| 125 | PULSE: Cycle the repeat state |
| 179 | ON: Set Random All On – provides feedback only OFF: Set Random All Off |
| 180 | ON: Set Random Off – provides feedback only OFF: Set Random On |
| 182 | ON: Set Repeat Track On – provides feedback only OFF: Set Repeat Track Off |
| 183 | ON: Set Repeat All On – provides feedback only OFF: Set Repeat All Off |
| 184 | ON: Set Repeat Off – provides feedback only OFF: Set Repeat On |
| 185 | PULSE: Frame Forward |
| 188 | PULSE: Slow Forward |
| 189 | PULSE: Slow Reverse |
| 192 | PULSE: Cycle Scan Speed |
| 241 | ON: Play is active – provides feedback only |
| 242 | ON: Stop is active – provides feedback only |
| 243 | ON: Pause is active – provides feedback only |
| 246 | ON: Scan Forward is active – provides feedback only |
| 247 | ON: Scan Reverse is active – provides feedback only |
| 249 | ON: Slow Forward is active – provides feedback only |
| 250 | ON: Slow Reverse is active – provides feedback only |
| 251 | ON: Device is Online – used for feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – use for feedback only OFF: Data is not Initialized |
| 255 | ON: Set power on – used for feedback also OFF: Set power off |
| *301 | PULSE: Feedback for Picture Mute |
| *302 | PULSE: Cycle Picture Mute |
| *303 | PULSE: Cycle Search |

Table 1 - Virtual Device Channel Events

## Command Control

The UI module controls the disc device via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| ?DEBUG | Request the state of the debug feature. ?DEBUG |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| ?DISCINFO | Get the information for the currently selected disc. ?DISCINFO |
| ?FWVERSION | Get the device’s firmware version. ?FWVERSION |
| PASSBACK-<state> | Enable or disable response reporting from the device. When enabled device responses will be sent as strings to the virtual device. Note: By default, this is set to off at startup. <state> : 0 = Off (default) 1 = On PASSBACK-0 |
| PASSTHRU-<string> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the Duet module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section below. <string> : string to send to unit PASSTHRU-PAG:1 (PAGE UP) Note:Do not send the start/end of character the module will append them before sending the command to the device. |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |
| TITLECOUNTERNOTIFY-<state> | Turn on or off the title counter notifications. <state> : 0 = Off 1 = On TITLECOUNTERNOTIFY-1 Note: - If notifications are turned on, they will be returned in the form of TITLECOUNTER-responses. - This is only applicable if the disc content has video. |
| ?TITLEINFO | Get the information for the currently selected title. ?TITLEINFO |
| TRACKCOUNTERNOTIFY-<state> | Turn on or off the track counter notifications. Note: If notifications are turned on, they will be returned in the form of TRACKCOUNTER- responses. <state> : 0 = Off 1 = On TRACKCOUNTERNOTIFY-1 Note: - If notifications are turned on, they will be returned in the form of TRACKCOUNTER-responses. - This is only applicable if the disc content is audio. |
| ?TRACKINFO | Get the current track’s information. ?TRACKINFO |
| ?VERSION | Query for the current version number of the Duet module. ?VERSION |
| ASPECT-<ratio> | Set the current aspect ratio. <ratio> : WIDESCREEN NORMAL ANAMORPHIC SQUEEZE ASPECT-NORMAL Note: WIDESCREEN is same as Letter Box NORMAL is same as Panscan ANAMORPHIC is same as Wide SQUEEZE is same as Squeeze |

Table 2 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for disc device changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| DEBUG-<value> | Returns the state of debugging messages in the UI module and the Comm. module. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and info messages on 4 = set all messages on DEBUG-1 |
| DISCINFO-<number>, <duration>, <#ofTitles>, <#ofTracks>, <discType>, <id> | Reports the information associated with the currently selected disc. <number> : 1 (disc number) <duration> : Not Supported (hh:mm:ss.ff)(hours:minutes:seconds:frames) <#ofTitles> : Not Supported <#ofTracks> : Not Supported <discType> : AUDIO_ONLY (audio content) VIDEO_ONLY (video content) AUDIO_VIDEO (audio and video content) UNKNOWN (unknown content) INVALID (no disc) <id> : Always Null for this device DISCINFO-1,--:--:--.--,-2147483648, 2147483648,AUDIO_VIDEO, NOTE: If you get -2147483648 for a value, this means that the parameter is not supported for the DV7600. |
| FWVERSION-<version> | Returns the device’s firmware version. <version> : xx.yy = firmware version number FWVERSION=1.01 |
| TITLECOUNTER-<counter> | Reports a change in the title counter. <counter> : [-]hh:mm:ss.ff String containing the current location within title (hours:minutes:seconds:frames) Note: This response is only returned if title counter notifications are turned on. Note: Only hh:mm:ss are reported, ff defaults to 00 TITLECOUNTER-01:05:03.00 |
| TITLEINFO-<number>, <duration>, <#ofTracks>, <discNumber> | Reports the information associated with the currently selected title. <number> : 1..n (Title number) <duration> : Not Supported(hh:mm:ss.ff) <#ofTracks> : Not Supported <discNumber> : 1 TITLEINFO-1,--:--:--.--,-2147483648,1 NOTE: If you get -2147483648 for a value, this means that the parameter is not supported for the DV7600. NOTE: The hh:mm:ss.ff format is the duration in hours:minutes:seconds.frames. |
| TRACKCOUNTER-<counter> | Reports a change in the track counter. <counter> : [-]hh:mm:ss.ff String containing the current location within track (hours:minutes:seconds:frames) Note: This response is only returned if track counter notifications are turned on. Note: Only mm:ss are reported, hh and ff default to 00 TRACKCOUNTER-00:05:03.00 |
| TRACKINFO-<number>, <duration>, <discNumber> | Reports information about the currently selected track. <number> : 1..n (Title number) <duration> : Not Supported(hh:mm:ss.ff) (hours:minutes:seconds:frames) <#ofTracks> : Not Supported <discNumber> : 1 TRACKINFO-1,--:--:--.--,1 |
| VERSION-<version> | Reports the version number of the module. <version> : xx.yy.zz = module version number VERSION-1.0.0 |

Table 3 - Command Feedback Definitions

## Device Notes

- The device will not be initialized in the standby(off) mode. The device has be in the operating mode(on) for the data to be initialized.

- Note that FWVERSION- returned is for RS-232 Ver.No.

- Pressing Pause after the first time will cause the DVD to "Step" (appears to be the same functionality as "Frame")

## Programming Notes

- This module de-queues commands to the receiver at a regular interval of 200 milliseconds.

- This module does not use polling since the device usually returns a response to most commands and provides asynchronous feedback for most changes made to the front panel.

- At startup and when the ‘REINIT’ command is used, all values are set to default values. If these values are not initialized during the startup or re-initialization sequence, then they remain set to their default values and may be returned if a query/get command is sent. The default value for all integers is –2147483648, which is the smallest valid integer available. Boolean values default to 0/false. Power defaults to ‘INVALID’.

- TITILENOTIFY AND TRACKNOTIFY are initialized to 0.

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the ‘PASSTHRU-‘ command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from the UI code to implement 'white balance adjustment'.

send_command vdvDevice, "'PASSTHRU-',$03,$10,$05,$14"

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)

### Responses from the device

The module will automatically interpret replies from the device and pass these on to the application code according to the documented API.  Some device replies may not be passed on to the application code.

To see all replies from the device unfiltered by the module, enable PASSBACK and use a DATA_EVENT with a string handler in the UI code.  Again, the device-specific protocol must be known in order to interpret these responses.  Even when PASSBACK is enabled, the module will still interpret device responses according to the standard API.